Internship Report

STUDY OF THE FRAUSCHER FADC R2 AXLE COUNTING SYSTEM AND DEVELOPMENT OF AN AUTOMATED TENDER DOCUMENT MANAGEMENT SYSTEM FOR RAILWAY STATION DETECTION POINTS

A report submitted

in partial fulfilment of the requirements

for the degree of

Bachelor of Technology (B.Tech)

by

Amrit Bhardwaj

(23EC008)

to the

School of Technology — Electronics and Communication Engineering

GATI SHAKTI VISHWAVIDYALAYA

Central University, Under the Ministry of Railways, Government of India

Vadodara, India – 390004

Date: 10 July 2026

Certificate of Completion

This is to certify that the work contained in the project titled "Study of the Frauscher FAdC R2 Axle Counting System and Development of an Automated Tender Document Management System for Railway Station Detection Points" has been successfully completed by Amrit Bhardwaj (Roll No. 23EC008) under my/our supervision during the Eight-Week Summer Internship (19 May 2026 to 19 July 2026) at Frauscher Sensor Technology (India). During the internship period, the student was found to be hardworking, punctual, and disciplined, and met all the criteria satisfactorily.

Date: 10 July 2026

Place: Vadodara, India

_________________________

[Faculty Guide Name], Faculty Guide, GSV Vadodara

| Student Name | Roll Number | Signature |
| --- | --- | --- |
| Amrit Bhardwaj | 23EC008 |  |

Industry Mentor Details

| Name | Signature |
| --- | --- |
| Mr. Dinesh Kumhar |  |
| Mr. Shiva Kumar |  |
| Mr. Amit Raj Gosavi |  |

Acknowledgement

I take this opportunity to express my sincere thanks to my mentors, Mr. Dinesh Kumhar, Mr. Shiva Kumar, Mr. Amit Raj Gosavi, for their guidance and support throughout the internship. I would also like to express my gratitude towards them for showing confidence in me. It was a privilege to have a great experience working under their supervision in a cordial environment.

I am very much thankful to [Faculty Guide Name] and Gati Shakti Vishwavidyalaya, Vadodara, for providing me the opportunity of pursuing my summer internship. I am also thankful to Frauscher Sensor Technology (India) for hosting the internship and providing the resources and infrastructure needed to complete this project.

I am thankful to the office staff and other relevant persons at both the University and the organisation for their continuous support during the internship period.

Amrit Bhardwaj

Abstract

This report presents the work carried out during an eight-week summer internship (19 May 2026 to 19 July 2026) at Frauscher Sensor Technology (India). The internship was structured into two phases. In the first phase, the intern was trained on the Frauscher Advanced Counter (FAdC R2), a SIL-4 certified axle counting system used in railway signalling for train occupancy detection. This phase covered the working principle of the RSR180 wheel sensor, the Axle Evaluation Board (AEB), the Communication Board (COM), the Power Supply with Crowbar Protection (PSC), the Input/Output Extension Board (IO-EXB), and the associated communication interfaces (Serial, CAN, Ethernet, and Hotlink). A comparative study of the FAdC R2 against conventional axle counting architectures, namely the Eldyne EAK/ACE system and the GGTronics High Availability Single Section Digital Axle Counter (HA-SSDAC), was also carried out to understand the evolution of axle counting technology in railway signalling.

In the second phase, the intern was assigned to the project management team and was involved in planning and developing a software system to automate the handling of tender documents across multiple railway station detection points. Tender documents were previously managed manually, which was slow and error-prone. To address this, the intern contributed to the design and development of a four-part automated pipeline: a Windows-based local agent that watches designated folders and performs resumable multipart uploads of tender documents to Amazon S3; an authenticated Node.js/Express and PostgreSQL backend that issues presigned upload URLs and tracks upload sessions; a PDF parsing service that extracts structured data from tender PDFs; and an Excel/dashboard processing pipeline that uses AI-assisted extraction to populate a live spreadsheet for the tender management team.

The internship provided practical exposure to railway signalling hardware, safety-critical system design, and full-stack software development, including cloud storage, authenticated APIs, background services, and AI-assisted document processing.

Contents

(Right-click the table above and select "Update Field" in Microsoft Word to populate page numbers.)

# 1. Introduction

## 1.1 Motivation

Railway signalling depends on the accurate and reliable detection of train presence to ensure safe train movement and prevent collisions. Axle counting systems, such as the Frauscher FAdC R2, form a critical part of this safety chain. Understanding the working of such a SIL-4 certified system provides valuable insight into how safety-critical hardware is designed, tested, and deployed in a real-world railway environment.

In parallel, the organisation handles a large volume of tender documents related to multiple railway station detection points. These documents were being managed manually, which was time-consuming, error-prone, and difficult to scale as the number of tenders increased. There was a clear need for an automated system that could receive, validate, upload, parse, and organise these documents with minimal manual effort. This motivated the second phase of the internship, in which the intern contributed to building such a system.

## 1.2 Objectives

- To understand the working principle of the Frauscher FAdC R2 axle counting system used in railway signalling.

- To study the RSR180 wheel sensor, AEB, COM board, PSC, and IO-EXB, along with their roles in train detection.

- To study the communication interfaces (Serial, CAN, Ethernet, Hotlink) used within the FAdC R2 architecture.

- To compare the FAdC R2 with conventional axle counting systems, namely Eldyne (EAK/ACE) and GGTronics HA-SSDAC.

- To identify the inefficiencies in the manual handling of tender documents for multiple station detection points.

- To design and develop an automated pipeline for tender document upload, validation, parsing, and dashboard generation.

- To gain practical exposure to full-stack development, cloud storage, background services, and AI-assisted data extraction.

## 1.3 Organization of Report

Chapter 2 describes the methodology followed during both the technical training phase and the project development phase. Chapter 3 presents the technical study of the Frauscher FAdC R2 axle counting system and its comparison with conventional systems. Chapter 4 presents the design, architecture, and implementation of the automated tender document management system developed during the project phase. Chapter 5 discusses the outcomes, implications, and recommendations arising from the internship. The report concludes with references, an appendix containing implementation details, and a week-wise overview of the internship activity.

## 1.4 Organization Information

Frauscher Sensor Technology (India) is part of the Frauscher Sensor Technology group, an Austrian company specialising in wheel sensors and axle counting systems for railway signalling and traffic management. Frauscher's products, including the RSR180 wheel sensor and the FAdC family of axle counters, are deployed globally for safe and reliable train detection. In India, the organisation supports railway signalling projects, including participation in tenders floated by Indian Railways for the supply and commissioning of axle counting equipment at various station detection points, which necessitated the internal tender-document management system developed during this internship.

# 2. Methodology

The internship followed a two-phase methodology combining structured technical training with hands-on project development.

## 2.1 Technical Training Phase

During the first two weeks, the intern studied the Frauscher FAdC R2 system through technical manuals, mentor-led explanations, and hands-on familiarisation with hardware components such as the RSR180 wheel sensor, AEB, COM board, PSC, and IO-EXB. Daily observations and explanations were recorded in a logbook. This was followed by a comparative study of alternative axle counting architectures (Eldyne and GGTronics HA-SSDAC) using reference documentation, to understand how the FAdC R2 architecture improves upon earlier designs.

## 2.2 Project Development Phase

For the remaining weeks, the intern worked with the project management team using an iterative, version-controlled development approach. Requirements were gathered from the tender management workflow, after which the system was broken down into four independent, loosely coupled components: a local Windows agent, a cloud backend, a PDF parsing service, and an Excel/dashboard processing pipeline. Each component was developed, tested, and integrated incrementally using Git for version control, with the backend and agent connected through an authenticated REST API. Automated tests (Vitest) were written for critical upload and authentication logic, and deployments were verified in a staging environment before integration.

# 3. Technical Study: Frauscher FAdC R2 Axle Counting System

## 3.1 Overview and Core Components

The Frauscher Advanced Counter (FAdC R2) is a SIL-4 certified axle counting system used in railway signalling to detect train occupancy. Instead of relying on track circuits, it counts the number of train axles entering and leaving a defined track section (FMA). If the number of axles entering equals the number leaving, the section is declared clear; otherwise, it remains occupied. This method provides reliable train detection even in harsh environmental conditions.

### 3.1.1 RSR180 Wheel Sensor

The RSR180 wheel sensor is mounted on the rail using the SK140 rail claw and detects every train wheel passing over it, generating signals that are sent to the AEB for processing. It operates on inductive sensing: transmitter and receiver coils inside the sensor housing create an electromagnetic field around the rail, which is disturbed when a steel wheel passes. The sensor contains two independent detection systems (Sensor A and Sensor B) positioned a fixed distance apart, allowing the AEB to determine the direction of train movement from the order in which each system is triggered.

### 3.1.2 SK140 Rail Claw

The SK140 rail claw mounts the RSR180 sensor onto the rail without drilling or welding, enabling fast installation and removal — useful during maintenance, tamping work, or sensor relocation.

### 3.1.3 Axle Evaluation Board (AEB)

The AEB supplies and evaluates the RSR180 wheel sensor, outputs digital counting-head information, counts axles, and generates fail-safe clear/occupied indications for up to two track sections (FMA). A single track section can consist of up to 16 counting heads, the hardware limit for one AEB.

### 3.1.4 Power Supply with Crowbar Protection (PSC)

The PSC provides a stable DC power supply (accepting 19 V DC to 72 V DC input) to all system components — the AEB, COM, and IO-EXB boards — inside the axle counter cubicle. Power distribution across backplanes is achieved through the X1 ("PWR IN") and X2 ("PWR OUT/IN") connectors, allowing multiple backplanes to be daisy-chained from a common source.

### 3.1.5 IO-EXB (Input/Output Extension Board)

The IO-EXB extends the input/output capability of an AEB, which can control up to eight IO-EXB boards. It does not require independent configuration and automatically inherits settings from its connected AEB. Depending on configuration, an IO-EXB performs one of three functions: Axle Counting Output (fail-safe clear/occupied relay contacts for up to two FMAs per board), Data Transmission (up to 24 digital input bits per AEB, transmitted over CAN), or Counting Head Output (direction and status information from connected counting heads).

The Clear/Occupied Output uses two relay outputs — Fm (Failsafe Clear) and Occ (Occupied) — wired so that all four Fm relay contacts must operate correctly before a clear indication is sent to the interlocking, preventing false-clear indications on relay or system failure.

### 3.1.6 COM Board and Backplane BP-PWR

The COM board is the communication module of the FAdC R2, receiving information from the AEB and forwarding track status (Clear/Occupied), diagnostic, and fault information to the interlocking and maintenance systems respectively, over two separate Ethernet ports. The backplane BP-PWR connects up to eight AEB and/or COM boards to a single PSC.

## 3.2 Communication Interfaces

The FAdC R2 provides four communication interfaces, each serving a distinct purpose:

- Serial Interface — an RJ45 diagnostic port on the AEB/COM front panel, used with service software (e.g. ASD) for maintenance and fault analysis; not to be confused with Ethernet despite the identical connector.

- CAN Interface — the internal communication backbone linking AEB and COM boards in a linear (non-branching) topology, limited to 20 m per segment, with dual redundant CAN paths for high availability.

- Ethernet Interface — located on the COM board, used for safety-related (track clear/occupied, axle count) and diagnostic communication with external interlocking and maintenance systems, using UDP/IP over 10BASE-T/100BASE-TX.

- Hotlink Interface — a shielded RJ45 link between two redundant COM boards (maximum 2 m crossover cable), enabling instant failover if the active COM board fails.

## 3.3 Comparative Study: Eldyne and GGTronics Systems

### 3.3.1 Eldyne System

The Eldyne system consists of outdoor trackside equipment (SK30H rail contacts and the Track-Side Electronic Unit, EAK) and indoor equipment (the Axle Counter Central Evaluator, ACE), connected by a communication link. The SK30H rail contact uses two sensor coils (Sk1, Sk2) to detect wheel passage and direction, and the EAK evaluates this information locally at the trackside before transmitting axle counts to the ACE. The ACE follows a 2-out-of-2 (2oo2) safety architecture with dual CPU cards that must agree before a decision is accepted. A key architectural difference from the FAdC R2 is that Eldyne performs evaluation at the trackside EAK, whereas the FAdC R2 performs evaluation indoors at the AEB, removing the EAK–ACE distance limitation imposed by the ISDN-based communication link.

### 3.3.2 GGTronics High Availability Single Section Digital Axle Counter (HA-SSDAC)

The HA-SSDAC, conforming to RDSO Specification RDSO/SPN/177/2013 Ver. 3.0, improves on the conventional Single Section Digital Axle Counter (SSDAC) through dual-system redundancy: two independent SSDAC systems are mounted on different rails, communicating through separate channels, with separate power supplies and parallel vital relays to avoid common-mode failures. It uses a fail-safe 2×(2-out-of-2) microcontroller architecture and Phase Detector (PD) cards that generate carrier frequencies (21 kHz and 25 kHz) for phase-reversal-based axle detection, with dual-frequency validation at the CPU before an axle count is registered.

This comparative study showed that while all three systems (FAdC R2, Eldyne, and GGTronics HA-SSDAC) achieve fail-safe axle counting through redundancy and cross-validation, they differ significantly in where evaluation is performed (indoor AEB vs. trackside EAK), the communication backbone used (CAN/Ethernet vs. ISDN), and the mechanism used for redundancy (COM board hotlink vs. dual SSDAC vs. dual CPU 2oo2).

# 4. Project Work: Automated Tender Document Management System

## 4.1 Problem Statement

Tender documents for multiple railway station detection points were being received and processed manually — collected on individual laptops, emailed, or copied by hand into spreadsheets — which was slow, inconsistent, and difficult to audit. The project management team required a system that would let a user simply drop a tender document into a folder, and have it validated, uploaded to secure cloud storage, parsed, and reflected in a shared dashboard without further manual effort.

## 4.2 System Architecture

The system was designed as four independent components connected through a central authenticated backend, so that each part could be developed, tested, and deployed on its own:

- Local Agent (Windows background service) — watches designated folders on a user's machine and uploads valid tender documents.

- Cloud Backend (Node.js/Express + PostgreSQL) — authenticates each machine, issues presigned upload URLs, and tracks upload sessions.

- PDF Parsing Service (Python) — extracts structured text and tender fields from uploaded PDF documents.

- Excel/Dashboard Processor (Node.js + Python) — uses AI-assisted extraction to populate a shared spreadsheet with parsed tender data.

## 4.3 Local Agent

The local agent is a Windows background service, built in TypeScript, that watches one or more shared folders using Chokidar. When a file is copied into a watched folder, the agent waits for the file to stop changing (a stability check), validates its extension and size, and moves invalid or oversized files to a rejected folder. Valid files are placed into a durable SQLite queue. A background worker then requests a multipart upload session from the backend and uploads the file directly to Amazon S3 in chunks using short-lived presigned URLs, saving each part's ETag so that an interrupted upload can resume from the last completed chunk rather than restarting. Network, timeout, and rate-limit failures are retried with exponential backoff, while authentication or configuration failures are placed in a durable "blocked" state pending recovery. The agent runs as a Windows service (via WinSW) so that end users only need to drop a file into a desktop "Upload Tender Documents" folder — no terminal, AWS account, or manual upload step is required. A companion desktop manager app (Electron) was also built for browsing and managing tender uploads.

## 4.4 Cloud Backend

The backend is an Express application backed by PostgreSQL, responsible for authenticating each agent machine, creating S3 multipart upload sessions, and issuing presigned part-upload URLs. AWS credentials exist only on the backend and are never sent to or stored on agent machines. Each machine authenticates using a unique, independently revocable API key, sent as a bearer token and verified against a SHA-256 hash stored in the database, so that raw keys are never persisted server-side. The backend exposes endpoints to initiate, upload parts for, complete, and abort a multipart upload, and uses an idempotency key on initiation to make retried requests safe. An administration CLI allows creating, listing, and revoking agent identities.

## 4.5 PDF Parsing and Excel/Dashboard Pipeline

Once a document reaches cloud storage, a Python-based PDF parsing service (built with the pdftext library, after evaluating a slower OCR-based approach) extracts the text content of each tender document. The extracted text is then passed through an AI-assisted extraction step using Google's Gemini 2.5 Flash Lite model, prompted to identify structured tender fields such as authority name, tender/work name, bidding type, and status. The extracted fields are written as a new row into a shared Excel workbook using the Google Sheets API, so that the tender management team always sees an up-to-date, centrally accessible record without needing to open or merge individual files.

## 4.6 Testing and Verification

Automated unit and integration tests (using Vitest) were written to cover valid, invalid, oversized, and still-growing files during folder watching; SQLite schema migration and queue-state transitions; retry scheduling and blocked-job recovery; multipart chunk boundaries and ETag persistence; and resume behaviour after an interrupted upload. Backend authentication and upload-service logic were similarly covered by unit and integration tests before deployment.

## 4.7 Deployment

The backend was deployed on Render, provisioned with a managed PostgreSQL database in the same region through an infrastructure blueprint (render.yaml). The PDF parsing service was containerised with Docker and deployed as a Hugging Face Space. The Windows agent was packaged into a self-contained distributable (bundled with its own Node.js runtime and native SQLite bindings) and installed on manager machines as a Windows service, with install/uninstall scripts and a production configuration template.

# 5. Implications and Recommendations

## 5.1 Outcomes

The internship resulted in a working understanding of a SIL-4 certified railway axle counting system and its safety architecture, as well as a functioning, tested pipeline that automates what was previously a manual, error-prone tender-document workflow across multiple station detection points. The pipeline removes manual data entry for tender records, gives the project management team a continuously up-to-date dashboard, and provides an auditable, resumable upload path for documents from user machines to secure cloud storage.

## 5.2 Implications

The comparative study of axle counting systems highlights a broader industry shift from trackside evaluation (Eldyne) and pure hardware redundancy (GGTronics HA-SSDAC) towards centralised indoor evaluation with software-configurable redundancy (Frauscher FAdC R2), reducing distance constraints and simplifying field maintenance. On the software side, the tender automation project demonstrates that even a relatively small, four-part system can meaningfully reduce manual effort when the components are kept independent, authenticated, and individually testable.

## 5.3 Recommendations

- Extend the AI-assisted extraction step with a validation/review step so that a human confirms low-confidence field extractions before they reach the dashboard.

- Add monitoring/alerting on the backend and agent so that blocked or failed uploads are surfaced to the project management team automatically rather than being discovered during manual checks.

- Move the free-tier Render database and web service to a paid plan before storing production tender data, since the free tier has no backups and can sleep after inactivity.

- Document the FAdC R2 vs. Eldyne vs. HA-SSDAC comparison as an internal reference note, since this knowledge is useful beyond the scope of this internship.

# References

[1] Frauscher Sensor Technology, "FAdC R2 — Frauscher Advanced Counter, Technical Manual," Frauscher Sensortechnik GmbH.

[2] Frauscher Sensor Technology, "RSR180 Wheel Sensor — Installation and Technical Data Sheet."

[3] Research Designs and Standards Organisation (RDSO), "RDSO/SPN/177/2013 Ver. 3.0 — Specification for High Availability Single Section Digital Axle Counter," Ministry of Railways, Government of India.

[4] Express.js Documentation, https://expressjs.com/.

[5] Amazon Web Services, "Uploading and copying objects using multipart upload," AWS S3 Developer Guide.

[6] Google, "Gemini API Documentation," Google AI for Developers.

[7] Google, "Google Sheets API Reference," Google for Developers.

# Appendix A. Repository Layout of the Tender Automation System

## A.1 Local Agent (TypeScript)

src/agent.ts — application entry point; src/watcher.ts — folder detection, stability checks, validation; src/queue.ts — durable SQLite queue and multipart state; src/uploader.ts — backend API calls and direct S3 part uploads; src/upload-worker.ts — background processing and retry policy; src/config.ts — YAML/environment validation; manager-app/ — Electron desktop tender download manager.

## A.2 Cloud Backend (Node.js / Express / PostgreSQL)

src/app.js — Express application and global middleware; src/config/ — environment and database setup; src/routes/, src/controllers/ — HTTP endpoints; src/services/upload.service.js, src/services/s3.service.js — multipart upload business rules and AWS SDK/presigning logic; src/repositories/ — PostgreSQL queries; src/middleware/, src/validators/ — authentication, validation, and error handling.

## A.3 PDF Parsing Service (Python)

app.py — Flask/Docker service that extracts text from tender PDFs using pdftext for near-instant extraction; deployed as a containerised Hugging Face Space.

## A.4 Excel / Dashboard Processor (Node.js + Python)

app.js, index.js — Express application; controllers/, routes/, services/ — S3 retrieval, parser invocation, and Excel/Google Sheets update logic; parser/parser.py — Python-based field parser invoked from the Node.js service; services/excelService.js — Google Sheets API integration for writing extracted tender rows.

# Weekly Overview of Internship Activity

| Week | Dates | Activity |
| --- | --- | --- |
| 1 | 19 – 25 May 2026 | Introduction to the Frauscher FAdC R2 axle counting system: working principle, RSR180 wheel sensor (inductive sensing, direction detection), SK140 rail claw, AEB, PSC, IO-EXB, COM board, and backplane BP-PWR. |
| 2 | 26 May – 1 Jun 2026 | Studied FAdC R2 communication interfaces (Serial, CAN, Ethernet, Hotlink) and power/protection architecture; comparative study of the Eldyne EAK/ACE system and the GGTronics HA-SSDAC against the FAdC R2. |
| 3 | 2 – 8 Jun 2026 | Shifted to the project management team; gathered requirements and planned an automated tender-document workflow for multiple station detection points; scaffolded the Tender Automation backend (Express/PostgreSQL). |
| 4 | 9 – 15 Jun 2026 | Built the local Windows agent: folder watching, file validation and rejection handling, durable SQLite queue, and its automated test suite; added database-backed agent authentication to the backend. |
| 5 | 16 – 22 Jun 2026 | Implemented resumable multipart S3 uploads in the agent and backend (presigned URLs, ETag persistence, retry/backoff), and refined logging, packaging, and the Windows-service setup. |
| 6 | 23 – 29 Jun 2026 | Developed the PDF parsing service (pdftext-based extraction) and the Excel/dashboard processor, including the initial Google Sheets API integration for writing tender rows automatically. |
| 7 | 30 Jun – 6 Jul 2026 | Integrated Gemini 2.5 Flash Lite for AI-assisted extraction of tender fields (authority name, work name, bidding type, status) from parsed PDF text; refined extraction prompts and regex-based field cleanup. |
| 8 | 7 – 13 Jul 2026 | Final integration and testing across all four components; new production database setup; built the Electron-based desktop tender manager app; deployment and verification (ongoing at report submission, internship concludes 19 Jul 2026). |

Note: Week boundaries above are inferred from project version-control history and mentor-guided training records, and should be adjusted to match the intern's personal logbook where they differ.